# STR-3221-R1 — Module for Smart Lighting & I/O Control

DIN rail • 24 V DC • Modbus RTU • Home Assistant

# Overview

See module README for details.

# Quick Overview

The **STR-3221-R1** is a compact and configurable **smart LED / I/O controller** designed for **multi-channel LED lighting and automation control** in architectural and industrial applications.

It provides **32 MOSFET-switched outputs** (open-drain, low-side, 12–24 V DC) for LED loads, along with **4 opto-isolated 24 V digital inputs** and **4 local buttons** each with indicator LEDs.

Configuration is handled through a browser-based **WebConfig interface** via **USB-C (Web Serial)** — no additional software or drivers are required.

The module communicates with a **MicroPLC/MiniPLC** or any Modbus master via **RS-485 (Modbus RTU)**, making it ideal for **staircase lighting, building automation, ambient illumination, alarm signaling, or other automation systems**.

Internally, it features robust isolation, surge protection, and power regulation circuits, ensuring reliable field operation and long service life.

**One-line purpose:** a **high-density field I/O lighting node** that’s easy to wire, configure, and supervise from PLC, SCADA, or home automation platforms.

---

# Typical Applications

These example illustrate how the **STR-3221-R1** can be integrated into real-world automation or lighting systems.

---

## Motion-Based Stair Lighting

**What it does:**

Automatically lights stair LEDs in sequence when motion is detected at the top or bottom of the staircase.

**Setup:**

Connect motion sensors to **IN1 (bottom)** and **IN2 (top)** terminals.

Connect each stair LED segment to outputs **O1–O32** (low-side switching).

Set WebConfig → **STR-3221-R1** .

Program MicoPLC/MiniPLC to poll **IN1/IN2** and activate LEDs in a timed sequence.

Use **Button 1** as “Manual Test / All ON” and **Button 2** as “All OFF”.

---

# Tech Specs

See module README for details.

# Installation, Environmental & Mechanical

See module README for details.

# Home Assistant & ESPHome Integration

See module README for details.

# Documentation

| Resource | Description |
| --- | --- |
| **🧠 Firmware (Arduino/PlatformIO)** | `Firmware/default_str_3221_r1/` — main sketch for Modbus, lighting, and WebConfig. |
| **🛠 WebConfig Tool** | `Firmware/ConfigToolPage.html` — browser-based USB-C setup utility. |
| **📷 Images & Diagrams** | `Images/` — module photos, terminal maps, and block diagrams. |
| **📐 Schematics (PDF)** | `Schematics/` — FieldBoard and MCUBoard schematics for hardware developers. |
| **📄 Datasheet & Manual** | `Manuals/` — module datasheet and installation guide. |
| **📦 Pre-built Firmware** | Releases page — compiled `.bin` files for direct flashing. |

---

# Power Supply

Describe 24 VDC input

List expected current

Explain isolated sensor power if present

# Inputs & Outputs

| Interface | Qty | Description |
| --- | --- | --- |
| **Digital Inputs** | 2 | Opto-isolated **24 VDC** via **ISO1212 (U23)**, surge-protected (F6/F7, D39). |
| **Outputs** | 32 | Low-side **MOSFET (SI2307A)** channels with **SS24** flyback diodes, grouped with shared **VCC** rails. |
| **Buttons** | 4 | Local control / override / test switches. |
| **Status LEDs** | 4 | User-assignable (power, activity, or logic indicator). |
| **RS-485 (Modbus RTU)** | 1 | Communication bus; **A/B/COM** terminals. |
| **USB-C (Setup Port)** | 1 | WebConfig / firmware interface (not for powering field devices). |
| **Power Input** | 1 | **24 VDC (V+, 0V)**; reverse and surge-protected; onboard 5 V / 3.3 V regulation. |
| **Sensor Rails (SENS.A / SENS.B)** | 2 pairs | Fused 24 V auxiliary rails for external sensors (low current). |

---

# Cable Recommendations & Shield Grounding

See module README for details.

# System Architecture & Pinout

Diagrams & Descriptions

<br>**System Block Diagram** — MCU, Modbus interface, power chain, and I/O groups.

**FieldBoard Layout** — 32 MOSFET outputs, ISO1212 inputs, 24 VDC power rails.

**MCU Board Layout** — RP2350 MCU, TLC59208F drivers, MAX485, and USB-C.

**PinOut** — Field wiring view with power, DI, outputs, and RS-485.

---

# ESPHome YAML Configuration & Hardware Pinout Manual

Diagrams & Descriptions

<br>**System Block Diagram** — MCU, Modbus interface, power chain, and I/O groups.

**FieldBoard Layout** — 32 MOSFET outputs, ISO1212 inputs, 24 VDC power rails.

**MCU Board Layout** — RP2350 MCU, TLC59208F drivers, MAX485, and USB-C.

**PinOut** — Field wiring view with power, DI, outputs, and RS-485.

---

# Links

**MicroPython**

**C / C++**

**Arduino IDE**

**PlatformIO**

The STR-3221-R1 firmware is compatible with standard RP2350 toolchains and examples.

It uses Modbus RTU libraries, Web Serial (for configuration), and I²C for LED drivers (TLC59208F).

---